# QA For barrel harnesses

Table 1 defines the electrical/optical performance items to be measured and the pass/fail criteria.

Table 1 Performance pass/fail criteria for barrel harnesses.

| Test Number | Item | Units | Min value | Max value | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Reflectance of 45 degree polished fibre | % | 80 | - | tbc |
|  | VCSEL total power @ 10 mA | W | 1000 | - | measured with large area photodiode |
|  | Decrease in power after burn-in | % |  | 20 |  |
|  | PIN responsivity | A/W | 0.4 |  |  |
|  | Decrease in pin/responsivity after burn-in | % |  | 20 |  |
|  | VCSEL coupled power @ 10 mA | W | 600 | - | temporary splice used for measurement |
|  | PIN responsivity @ -5V | A/W | 0.4 | - | temporary splice used for measurement |
|  | VCSEL power for opto-package on dogleg. Decrease cf test 6 | % | 30 | - | AC measurement. If fail, check with DC. |
|  | Repeat 8 after burn-in. Decrease cf test 8 | % | 20 |  |  |
|  | PIN responsivity for otpo-package on dogleg. Change cf test 7 | % | 20 | - | AC measurement. If fail, check with DC |
|  | Repeat 10 after burn-in. decrease cf test 10 | % | 20 | - |  |
|  | data link eye pattern | number of events in bad region of mask for 30 seconds |  | 0 | mask defined by 20%-80% signal and rise/fall time 1 ns and jitter 1ns. |
|  | ttc ber test | number of errors in 10s |  | 0 | VCSEL power set to ~ nominal 1000 w. |
|  | power tape assembly resistance between PP1-N and i/f PCB for power lines (signal lines). | m | - | 0.144 (1.344) | Allows for +/-15% tolerance. |
|  | insulation between traces | Mm | - | 1 |  |
|  | insulation resistance test on HV line | - |  |  | test performed at 500v. |
|  | VCSEL power on harness. decrease cf test 9 | % | - | 20 |  |
|  | PIN responsivity. decrease cf test 11` | % | - | 20 |  |
|  | BER test data link | No error in 1 hour or less than 1 error per hour in 3 hour run. |  |  | VCSEL power set to ~ nominal 1000 w. |
|  | BER test data link | No error in 1 hour or less than 1 error per hour in 3 hour run. |  |  | VCSEL current set to 10 mA. |

The mechanical QA shall consist of the measurements defined in Table 2 below.

Table 2 Mechanical QA

| Parameter | Maximum Value (mm) | Comments |
| --- | --- | --- |
| Thickness Al/kapton sheet | 0.110 | Measured by Ljublajana on receipt of sheet. Allows for +/- 10% spec. |
| Thickness of LMT pair | 0.289 | Measured by Ljublajana. Allows for +/- 10% spec. |
| Thickenss of 6 pairs of LMTs | 1.733 | Measured by Ljublajana. Allows for +/- 10% spec. |
| Height of harness stack for interface PCB for end dogleg. | 5.7 | Measured by Taiwan after harness assembly. |
| Height above barrel surface of harness stack for interface PCB for end dogleg. | 6.0 | Allows 300 m tolerance. Measured by RAL after harness mounted on barrel. |
| Thickness AlN ceramic | 0.406 | Allows 25 m tolerance. Measured by RAL before glueing to dogleg |
| Dogleg thickness under opto-package | 0.34 | Allows for +/- 10% spec. Measured by Taiwan. |
| AlN+ thermally conductive glue + dogleg | 0.846 | Allows for 100 m of glue layer. |
| Opto-package and chip cover + glue | 1.6 | Measured by Taiwan |
| Total height of AlN + dogleg + opto-package above flat surface. | 2.45 | Allows for above tolerances. Measured by Taiwan. |