Author: Tadej MALI

Figure number: 1

Orietation: portrait

Figure 1: Silicon microstrip detector used in the ‘edge-on’ geometry. X-ray photons emerge from the source on the right hand side of the figure.

Author: Tadej MALI

Figure number: 2

Orietation: portrait

Figure 2: X-ray imaging setup. An object was positioned on the moving stage, while the position of the X-ray source and detector was fixed. An image of a narrow (3.1 mm in y direction) region of the object is obtained by scanning in x direction.

Author: Tadej MALI

Figure number: 3

Orietation: portrait

Figure 3: Microcalcifications with a diameter of 350 m. The contrast of the object at x=3.5 is 6%. The image was obtained by scanning in x direction with a scanning step of 200 m; the strip pitch of the detector was 100 m. The size of pixels is 300×100 m2 (x × y size). The image was obtained at a skin entrance dose of 1 mGy.

In Fig. 3a three microcalcifications can be seen at positions: (x=6, y=0.25), (x=3.5, y=2.15) and (x=9.2, 1.8). A cut through Fig. 3a at y=2.15 is shown in Fig. 3b.

Author: Tadej MALI

Figure number: 4

Orietation: portrait

Figure 4: Cluster of microcalcifications with diameter of 350 m imaged at different level of exposure. Fig. 4a: 1 mGy, Fig. 4b: 0.5 mGy, Fig. 4c: 0.25 mGy and Fig. 4d 0.16 mGy. Microcalcifications are clearly visible at skin entrance dose of 0.25 mGy and can still be detected at a dose of 0.16 mGy.

Author: Tadej MALI

Figure number: 5

Orietation: portrait

Figure 5: The 5 lp/mm detail from the phantom (Fig. 5a). Due to relatively large, finite size of pixel in x direction (300 m), the image is distorted. Instead of five lines, separated by 200 m, there are four lines separated by 200 m, superimposed on a plateau (see text below). A x direction cut of Fig. 5a at y = 0.5 mm is shown in Fig. 5b.